# HP Insight Manager WBEM Fibre Channel Host Bus Adapter

Description

The HP Insight Management Web-Based Enterprise Management (WBEM) Fibre Channel Host Bus Adapter Profile describes the behavior of Fibre Channel host adapters supporting the SCSI (Fibre Channel Protocol) command set. A Fibre Channel (FC) adapter used in a host system is called a Host Bus Adapter (HBA). An HBA is a physical device that contains one or more Fibre Channel ports. A single system can contain one or more HBAs.

This provider implements the following profiles and installs the necessary files:

| Profile Name | Organization | Version |
| --- | --- | --- |
| SNIA SMI-S FC HBA Profile, Host book | Storage Networking Industry Association (SNIA) | 1.4.0 r1 |
| HP FC HBA Profile | HP WBEM TC | 1.0 |

| Requirements | For each hardware architecture listed, this provider requires the following distributions HP Integrity managed servers; SLES 10 and later RLEL 5.0 and later. HP ProLiant managed servers; SLES 11 and later RHEL 5.3 and later. |
| --- | --- |
| Release History | Initial release with HP Insight Management Providers for Linux v2.0. |

## Setting Up the Provider

| Installing the Provider | There are no special installation instructions for this provider. It is installed by default as part of the HP Insight Management WBEM providers. |
| --- | --- |
| Configuring the Provider | This provider does not accept specific configuration adjustments beyond standard HP Insight Management WBEM support. |

## Using the Provider

| Namespaces Supported by the Provider Schema Supported by the Provider | This provider returns instances in the root/hpq namespace. |
| --- | --- |
|  | This provider supports the following classes: - SMX_FCHBAFCPort - SMX_FCHBAPortController - SMX_FCHBAFCPortStatistics - SMX_FCHBALogicalPortGroup - SMX_FCHBAPhysicalPackage - SMX_FCHBAProduct - SMX_FCHBASoftwareIdentityDrv - SMX_FCHBAFirmwareIdentityFW - SMX_FCHBAFirmwareIdentityBIOS - SMX_FCHBAGroupSystemSpecificCollection - SMX_FCHBAComputerSystemFCPort - SMX_FCHBAComputerSystemPortController - SMX_FCHBAComputerSystemLogicalPortGroup - SMX_FCHBAComputerSystemSoftwareIdentityDrv - SMX_FCHBAFCPortFCPortStatistics - SMX_FCHBAFCPortLogicalPortGroup - SMX_FCHBAPortControllerFCPort - SMX_FCHBAPhysicalPackagePortController - SMX_FCHBASoftwareIdentityDrvPortController - SMX_FCHBAPortControllerFirmwareIdentityFW - SMX_FCHBAPortControllerFirmwareIdentityBIOS - SMX_FCHBAProductPhysicalComponent - SMX_FCHBASystemPackaging - SMX_FCHBAGroupHostedCollection - SMX_FCHBAFCPortGroupSystemSpecific - SMX_FCHBALocation - SMX_FCHBAPhysicalElementLocation |

The tables in the following sections describe the properties of the supported classes. The classes are categorized by the class or superclass that defines the property, the first column is the Property Name (including type and units) and the second column describes how the provider determines the properties implementation. When the Property Implementation value is a number, the number given is the default behavior and the Managed Object Format interpretation is within parenthesis. If other values are returned, a problem is indicated.

Unless otherwise noted, all of the property implementation values given are for HP ProLiant and HP Integrity (cellular and non-cellular) systems. The location related properties and implementation values are determined based on the server type so they may differ.

### SMX_FCHBAComputerSystem

SMX_FCHBAComputerSystem is used to idenitfy classes that represent all Fibre Channel ports.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Fibre Channel System |
| ElementName | Fibre Channel System |
| CIM_ManagedSystemElement |  |
| Name | GUID |
| OperationalStatus[0] | - 0 (Unknown) - 2 (OK) - 6 (Error) - 10 (Stopped) |
| StatusDescriptions[0] | - Unknown - OK - Stopped - Error |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_Systems |  |
| CreationClassName | SMX_FCHBAComputerSystem |
| NameFormat | Other |
| CIM_ComputerSystem |  |
| Dedicated | 0 |
| IdentifyingDescriptions[0] | HPQ:GUID |
| OtherIdentifyingInfo[0] | GUID |

### SMX_FCHBAFCPort Class

The SMX_FCHBAFCPort class implements the HPFCHBA_FCPort class is used to represent a FC port device.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Fibre Channel Port <port controller number> <-port number> Where: <FC port controller number> is the associated FC port controller instance number; <-port number> is the FC port number and is only added when the associated FC port controller has multiple FC ports. |
| CIM_ManagedSystemElement |  |
| OperationalStatus[0] | 0 (Unknown) 2 (OK-Online) 6 (Error) 10 (Stopped) 11 (In Service) |
| StatusDescriptions[0] | Unknown OK-Online Stopped Offline Stopped Bypassed Stopped Diagnostics Stopped Link down Error |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_LogicalDevice |  |
| CreationClassName | SMX_FCHBAFCPort |
| DeviceID | Opaque value. |
| SystemName | Opaque value. |
| SystemCreationClassName | SMX_FCHBAComputerSystem |
| CIM_LogicalPort |  |
| MaxSpeed | One of (in Bits per Second): 0 1000000000 2000000000 10000000000 |
| OtherPortType | Other Not present Fabric Public Loop FL Port Fabric Port Fabric expansion port Generic Fabric Port Private Loop Point to Point |
| PortType | 1 (Other) |
| Speed | One of (in Bits per Second): 0 - 1000000000 2000000000 10000000000 |
| UsageRestriction | 4 (not restricted) |
| CIM_NetworkPort |  |
| LinkTechnology | 4 (FC) |
| PermanentAddress | Port WWN |
| PortNumber | Port number. |
| SupportedMaximumTransmissionUnit | Port maximum frame size. |

### SMX_FCHBAPortController Class

The SMX_FCHBAPortController class represents a logical device corresponding to a hardware network port controller.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Product Name |
| CIM_ManagedSystemElement |  |
| Name | NodeWWN |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_LogicalDevice |  |
| CreationClassName | SMX_FCHBAPortController |
| DeviceID | Node WWN:SerialNumber:<instanceNum> Where: <instanceNum> is the port controller instance number for the same adapter. |
| SystemCreationClassName | SMX_FCHBAComputerSystem |
| SystemName | Opaque |
| CIM_Controller |  |
| CIM_PortController |  |
| ControllerType | 4 (FC) |

### SMX_FCHBAFCPortStatistics Class

The SMX_FCHBAFCPortStatistics class represents statistics for a FC port. .

The following table lists the properties implemented and the HBAAPI_PORTSTATISTICS prefix refers to the HBA API HBA_PORTSTATISTICS structure. This information comes directly from the HBA and is not altered.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Product name |
| CIM_StatisticalData |  |
| InstanceID | Opaque |
| SampleInterval | 0 |
| CIM_NetworkPortStatistics |  |
| BytesTransmitted | HBAAPI_PORTSTATISTICS.TxWords |
| BytesReceived | HBAAPI_PORTSTATISTICS.RxWords |
| PacketsTransmitted | HBAAPI_PORTSTATISTICS.TxFrames |
| PacketsReceived | HBAAPI_PORTSTATISTICS.RxFrames |
| CIM_FCPortStatistics |  |
| CRCErrors | HBAAPI_PORTSTATISTICS.InvalidCRCCount |
| DumpedFrames | HBAAPI_PORTSTATISTICS.DumpedFrames |
| ErrorFrames | HBAAPI_PORTSTATISTICS.ErrorFrames |
| InvalidTransmissionWords | HBAAPI_PORTSTATISTICS.InvalidTxWordCount |
| LinkFailures | HBAAPI_PORTSTATISTICS.LinkFailureCount |
| LIPCount | HBAAPI_PORTSTATISTICS.LIPCount |
| LossOfSignalCounter | HBAAPI_PORTSTATISTICS.LossOfSignalCount |
| LossOfSyncCounter | HBAAPI_PORTSTATISTICS.LossOfSyncCount |
| NOSCount | HBAAPI_PORTSTATISTICS.NOSCount |
| PrimitiveSeqProtocolErrCount | HBAAPI_PORTSTATISTICS.PrimitiveSeqProtocolErrCount |

### SMX_FCHBALogicalPortGroup Class

The SMX_FCHBALogicalPortGroup class is a collection of one or more ports logically grouped for administrative and discovery/topology purposes.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| Description | Grouping of all FC Ports for this HBA |
| Caption | Logical Port Group |
| ElementName | Logical Port Group |
| CIM_Collection |  |
| CIM_SystemSpecificCollection |  |
| InstanceID | Opaque |
| CIM_LogicalPortGroup |  |
| Name | NodeWWN |
| NameFormat | WWN |

### SMX_FCHBAPhysicalPackage Class

The SMX_FCHBAPhysicalPackage class represents the physical elements of the associated FC port controller.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Product Name |
| CIM_ManagedSystemElement |  |
| Name | NodeWWN |
| OperationalStatus[] | 1 (Other) 2 (OK) 3 (Degraded) 6 (Error) |
| CIM_PhysicalElement |  |
| CreationClassName | SMX_FCHBAPhysicalPackage |
| Manufacturer | Manufacturer |
| Model | Model |
| OtherIdentifyingInfo | Location; PCI Slot <x> Where: <x> is the PCI slot number OR PCI Mezzanine=<x> where <x> is the Mexannine slot number. |
| SerialNumber | Serial Number |
| Tag | NodeWWN: <instanceNum> Where: <instanceNum> is the port controller instance number for the same adapter |
| Version | Board Version or N/A when not available |
| CIM_PhysicalPackage |  |
| RemovalConditions | 2 (Not Applicable) |

### SMX_FCHBAProduct Class

The SMX_FCHBAProduct class is used to aggregate physical element of the FC HBA.

The following table lists the properties implemented and the HBAAPI_ADAPTERATTRIBUTES prefix refers to the HBA API HBA_ADAPTERATTRIBUTES structure. This information comes directly for the HBA and is not altered

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | Product Name |
| CIM_Product |  |
| IdentifyingNumber | HBAAPI_ADAPTERATTRIBUTES.VendorSpecificID |
| Name | NodeWWN: <instanceNum> Where: <instanceNum> is the port controller instance number for the same adapter |
| Vendor | Manufacturer |
| Version | F/W Version of HBA or N/A when not available |

### SMX_FCHBASoftwareIdentityDrv Class

The SMX_FCHBASoftwareIdentityDrv class represents adapter software associated with the FC port controller.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | FCHBA Driver |
| Description | <Product_Name> Device Driver Where: <Product_Name> is the product name of the HBA. |
| ElementName | FCHBA Driver |
| CIM_ManagedSystemElement |  |
| Name | NodeWWN |
| CIM_LogicalElement |  |
| CIM_SoftwareIdentity |  |
| ClassificationDescriptions | FCHBA Driver |
| Classifications | 2 (Driver) |
| InstanceID | HPQ:NodeWWN:<instanceNum> Where: <instanceNum> is the port controller instance number for the same adapter |
| Manufacturer | Manufacturer |
| VersionString | File version of the Driver. |

### SMX_FCHBAFirmwareIdentityFW Class

The SMX_FCHBAFirmwareIdentityFW class represents adapter firmware associated with the FC port controller.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | FCHBA Firmware |
| Description | FCHBA Firmware details for <Model_Name> Where: <Model_Name> is the model name of the HBA. |
| ElementName | FCHBA Firmware |
| CIM_ManagedSystemElement |  |
| Name | NodeWWN |
| CIM_LogicalElement |  |
| CIM_SoftwareIdentity |  |
| ClassificationDescriptions | FCHBA Firmware |
| Classifications | 10 (firmware) |
| InstanceID | HPQ:NodeWWN:<instanceNum> Where: <instanceNum> is the port controller instance number for the same adapter |
| Manufacturer | Manufacturer |
| SerialNumber | Serial number |
| VersionString | Firmware version |

### SMX_FCHBAFirmwareIdentityBIOS Class

The SMX_FCHBAFirmwareIdentityBIOS class represents adapter BIOS associated with the FC port controller.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | FCHBA Option ROM |
| Description | FCHBA Option ROM detail for <Model_Name> Where: product name is the Model_Name of the HBA. |
| ElementName | FCHBA Option ROM |
| CIM_ManagedSystemElement |  |
| Name | NodeWWN |
| CIM_LogicalElement |  |
| CIM_SoftwareIdentity |  |
| ClassificationDescriptions | FCHBA Option ROM |
| Classifications | 11 (bios/fc) |
| InstanceID | HPQ:NodeWWN:<instanceNum> Where: <instanceNum> is the port controller instance number for the same adapter |
| Manufacturer | Manufacturer |
| VersionString | BIOS Version as reported by HBA |
| TargetTypes | HBA BIOS UNKNOWN |
| SerialNumber | Serial Number |
| MajorVersion | 0 |
| MinorVersion | 0 |
| RevisionNumber | 0 |
| HP_SoftwareIdentity |  |

### SMX_FCHBAGroupSystemSpecificCollection Class

The SMX_FCHBAGroupSystemSpecificCollection class is used to represent the group status of all FC Ports.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | FCHBA Consolidated Status |
| Description | Consolidated status for HP FC HBAs |
| ElementName | FCHBA Consolidated Status |
| CIM_Collection |  |
| CIM_SystemSpecificCollection |  |
| InstanceID | Opaque |
| HP_GroupSystemSpecificCollection |  |
| GroupOperationalStatus[0] | 0 (Unknown) 1 (Other) 2 (OK) 3 (Degraded) 6 (Error) 10 (Stopped) |
| GroupStatusDescriptions[0] | Unknown Other OK Degraded Error Stopped |

### SMX_FCHBAComputerSystemFCPort Class

The SMX_FCHBAComputerSystemFCPort class is used to represent an association between SMX_ComputerSystem and HPFCHBA_FCPort.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Component |  |
| CIM_SystemDevice |  |
| SMX_FCHBAComputerSystemFCPort |  |
| GroupComponent | Refer to SMX_FCHBAComputerSystem class properties. |
| PartComponent | Refer to SMX_FCHBAFCPort class properties. |

### SMX_FCHBAComputerSystemPortController Class

The SMX_FCHBAComputerSystemPortController class is used to represent an association between SMX_ComputerSystem and SMX_FCHBAPortController.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Component |  |
| CIM_SystemDevice |  |
| SMX_FCHBAComputerSystemPortController |  |
| GroupComponent | Refer to SMX_FCHBAComputerSystem class properties. |
| PartComponent | Refer to SMX_FCHBAPortController class properties. |

### SMX_FCHBAComputerSystemLogicalPortGroup Class

The SMX_FCHBAComputerSystemLogicalPortGroup class is used to represent an association between SMX_ComputerSystem and SMX_FCHBALogicalPortGroup.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Dependency |  |
| CIM_HostedDependency |  |
| CIM_HostedCollection |  |
| SMX_FCHBAComputerSystemLogicalPortGroup |  |
| Antecedent | Refer to SMX_FCHBAComputerSystem class properties. |
| Dependent | Refer to SMX_FCHBALogicalPortGroup class properties. |

### SMX_FCHBAComputerSystemSoftwareIdentityDrv Class

The SMX_FCHBAComputerSystemSoftwareIdentityDrv class is used to represent an association between SMX_ComputerSystem and SMX_FCHBASoftwareIdentityDrv.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Component |  |
| CIM_SystemDevice |  |
| SMX_FCHBAComputerSystemSoftwareIdentityDrv |  |
| InstalledSoftware | Refer to SMX_FCHBASoftwareIdentityDrv class properties. |
| System | Refer to SMX_FCHBAComputerSystem class properties. |

### SMX_FCHBAFCPortFCPortStatistics Class

The SMX_FCHBAFCPortFCPortStatistics class is used to represent an association between SMX_FCHBAFCPort and SMX_FCHBAFCPortStatistics.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_ElementStatisticalData |  |
| SMX_FCHBAFCPortFCPortStatistics |  |
| ManagedElement | Refer to SMX_FCHBAFCPort class properties. |
| Stats | Refer to SMX_FCHBAFCPortStatistics class properties. |

### SMX_FCHBAFCPortLogicalPortGroup Class

The SMX_FCHBAFCPortLogicalPortGroup class is used to represent an association between SMX_FCHBAFCPort and SMX_FCHBALogicalPortGroup.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_MemberOfCollection |  |
| SMX_FCHBAFCPortLogicalPortGroup |  |
| Collection | Refer to SMX_FCHBALogicalPortGroup class properties. |
| Member | Refer to SMX_FCHBAFCPort class properties. |

### SMX_FCHBAPortControllerFCPort Class

The SMX_FCHBAPortControllerFCPort is used to represent an association between SMX_FCHBAPortController and SMX_FCHBAFCPort.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Dependency |  |
| CIM_ControlledBy |  |
| SMX_FCHBAPortControllerFCPort |  |
| Antecedent | Refer to SMX_FCHBAPortController class properties. |
| Dependent | Refer to SMX_FCHBAFCPort class properties. |

### SMX_FCHBAPhysicalPackagePortController Class

The SMX_FCHBAPhysicalPackagePortController class is used to represent an association between SMX_FCHBAPortController and SMX_FCHBAPhysicalPackage.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Dependency |  |
| CIM_Realizes |  |
| SMX_FCHBAPhysicalPackagePortController |  |
| Antecedent | Refer to SMX_FCHBAPhysicalPackage class properties. |
| Dependent | Refer to SMX_FCHBAPortController class properties. |

### SMX_FCHBASoftwareIdentityDrvPortController Class

The SMX_FCHBASoftwareIdentityDrvPortController class is used to represent an association between SMX_FCHBAPortController and SMX_FCHBASoftwareIdentityDrv.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Dependency |  |
| CIM_ElementSoftwareIdentity |  |
| SMX_FCHBASoftwareIdentityDrvPortController |  |
| Antecedent | Refer to SMX_FCHBASoftwareIdentityDrv class properties. |
| Dependent | Refer to SMX_FCHBAPortController class properties. |

### SMX_FCHBAPortControllerFirmwareIdentityFW Class

The SMX_FCHBAPortControllerFirmwareIdentityFW class is used to represent an association between SMX_FCHBAPortController and SMX_FCHBAFirmwareIdentityFW.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Dependency |  |
| CIM_ElementSoftwareIdentity |  |
| SMX_FCHBAPortControllerFirmwareIdentityFW |  |
| Antecedent | Refer to SMX_FCHBAFirmwareIdentityFW class properties. |
| Dependent | Refer to SMX_FCHBAPortController class properties. |

### SMX_FCHBAPortControllerFirmwareIdentityBIOS Class

The SMX_FCHBAPortControllerFirmwareIdentityBIOS class is used to represent an association between SMX_FCHBAPortController and SMX_FCHBAFirmwareIdentityBIOS.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Dependency |  |
| CIM_ElementSoftwareIdentity |  |
| SMX_FCHBAPortControllerFirmwareIdentityBIOS |  |
| Antecedent | Refer to SMX_FCHBAFirmwareIdentityBIOS class properties. |
| Dependent | Refer to SMX_HPFCHBAPortController class properties. |

### SMX_FCHBAProductPhysicalComponent Class

The SMX_FCHBAProductPhysicalComponent class is used to represent an association between SMX_FCHBAProduct and SMX_FCHBAPhysicalPackage.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Component |  |
| CIM_ProductPhysicalComponent |  |
| SMX_FCHBAProductPhysicalComponent |  |
| GroupComponent | Refer to SMX_FCHBAProduct class properties. |
| PartComponent | Refer to SMX_FCHBAPhysicalPackage class properties. |

### SMX_FCHBASystemPackaging Class

The SMX_FCHBASystemPackaging class is used to represent an association between SMX_FCHBAPhysicalPackage and SMX_ComputerSystem.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Dependency |  |
| CIM_SystemPackaging |  |
| SMX_FCHBASystemPackaging |  |
| Antecedent | Refer to SMX_FCHBAPhysicalPackage class properties. |
| Dependent | Refer to SMX_FCHBAComputerSystem class properties. |

### SMX_FCHBAGroupHostedCollection Class

The SMX_FCHBAGroupHostedCollection class is used to represent an association between SMX_ComputerSystem and SMX_FCHBAGroupSystemSpecificCollection.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_Dependency |  |
| CIM_HostedDependency |  |
| CIM_HostedCollection |  |
| HP_GroupHostedCollection |  |
| SMX_FCHBAGroupHostedCollection |  |
| Antecedent | Refer to SMX_ComputerSystem class properties. |
| Dependent | Refer to SMX_FCHBAGroupSystemSpecificCollection class properties. |

### SMX_FCHBAFCPortGroupSystemSpecific Class

The SMX_FCHBAFCPortGroupSystemSpecific class is used to represent an association between SMX_FCHBAFCPort and SMX_FCHBAGroupSystemSpecificCollection.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_MemberOfCollection |  |
| SMX_FCHBAFCPortGroupSystemSpecific |  |
| Collection | Refer to SMX_FCHBAGroupSystemSpecificCollection class properties. |
| Member | Refer to SMX_FCHBAFCPort class properties. |

### SMX_FCHBALocation Class

The SMX_FCHBALocation class is used to specify the location of the HBA, PCI Slot number.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| CIM_Location |  |
| Name | NodeWWN:SerialNumber :<instanceNum> Where: <instanceNum> is the port controller instance number for the same adapter. |
| HP_Location |  |
| ElementLocationTag | <physical location> Refer to “Physical Location” for more information. |
| ElementLocationTagDesc | 1 (slot) |
| PhysicalPosition | <physical location> Refer to “Physical Location” for more information. |

### SMX_FCHBAPhysicalElementLocation Class

The SMX_FCHBAPhysicalElementLocation is used to represent an association between the SMX_FCHBAPhysicalPackage and SMX_FCHBALocation.

The following table lists the properties implemented.

| Property Name | Value |
| --- | --- |
| CIM_ElementLocation |  |
| SMX_FCHBAPhysicalElementLocation |  |
| Element | Refer to SMX_FCHBAPhysicalPackage class properties. |
| PhysicalLocation | Refer to SMX_FCHBALocation class properties. |

### SMX_FCHBASCSIProtocolController

SMX_FCHBASCSIProtocolController represents a protocol controller that manages a SCSI interface. This class is used for backward compatibility with SMI-S 1.0.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | ProductName |
| CIM_ManagedSystemElement |  |
| CIM_SystemSpecificCollection |  |
| OperationalStatus[0] | 0 (Unknown) 2 (OK-Online) 6 (Error) 10 (Stopped) 11 (In Service) |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_LogicalDevice |  |
| CreatingClassName | SMX_FCHBASCSIProtocolController |
| DeviceID | PortWWN |
| SystemName | Opaque value |
| SystemCreationClassName | SMX_FCHBAComputerSystem |
| CIM_ProtocolController |  |
| MaxUnitsControlled | 255 |
| CIM_SCSIProtocolController |  |

### SMX_FCHBASCSIProtocolEndpoint

SMX_FCHBASCSIProtocolEndpoint represents a SCSI protocol controller running through a part.

| Property Name | Value |
| --- | --- |
| CIM_ManagedElement |  |
| ElementName | SCSI Protocol Endpoint |
| CIM_ManagedSystemElement |  |
| Name | PortWWN |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| RequestedState | 12 (Not Applicable) |
| CIM_ServiceAccessPoint |  |
| CreatingClassName | SMX_FCHBASCSIProtocolEndpoint |
| SystemName | Opaque value |
| SystemCreationClassName | SMX_FCHBAComputerSystem |
| CIM_ProtocolEndpoint |  |
| ProtocolFType | 57 (HIPPI Interface) |
| CIM_SCSIProtocolController |  |
| ConnectionType | 2 (fibre channel) |
| Role | 2 (initiator) |

### SMX_FCHBAComputerSystemSCSIProtocolController

SMX_FCHBAComputerSystemSCSIProtocolController is used to represent an association between SMX_FCHBAComputerSystem and SMX_FCHBASCSIProtocolController.

| Property Name | Value |
| --- | --- |
| CIM_Component |  |
| CIM_SystemDevice |  |
| SMX_FCHBAComputerSystemSCSIProtocolController |  |
| GroupComponent | ref:SMX_FCHBAComputerSystem |
| PartComponent | ref:SMX_FCHBASCSIProtocolController |

### SMX_FCHBAFCPortSCSIProtocolEndpoint

SMX_FCHBAFCPortSCSIProtocolEndpoint is used to represent an association between SMX_FCHBAFCPort and SMX_FCHBASCSIProtocolEndpoint.

| Property Name | Value |
| --- | --- |
| CIM_Dependency |  |
| CIM_DeviceSAPImplementation |  |
| SMX_FCHBAFCPortSCSIProtocolEndpoint |  |
| Antecendent | ref:SMX_FCHBAPort |
| Dependent | ref:SMX_FCHBASCSIProtocolEndpoint |

### SMX_FCHBASCSIProtocolControllerFCPort

SMX_FCHBASCSIProtocolControllerFCPort is used to represent an association between SMX_FCHBASCSIProtocolController and SMX_FCHBAFCPort.

| Property Name | Value |
| --- | --- |
| CIM_Dependency |  |
| CIM_ProtocolControllerForDevice |  |
| CIM_ProtocolControllerFCPort |  |
| SMX_FCHBASCSIProtocolControllerFCPort |  |
| Antecendent | ref:SMX_FCHBASCSIProtocolController |
| Dependent | ref:SMX_FCHBAFCPort |

### SMX_FCHBASystemServiceAccessPoint

SMX_FCHBASystremServiceAccessPoint is used to represent an association between SMX_FCHBAComputerSystem and SMX_FCHBASCSIProtocolEndpoint.

| Property Name | Value |
| --- | --- |
| CIM_Dependency |  |
| CIM_HostedDependency |  |
| CIM_HostedAccessPoint |  |
| SMX_FCHBASysteServiceAccessPoint |  |
| Antecendent | ref:SMX_FCHBAComputerSystem |
| Dependent | ref:SMX_ FCHBASCSIProtocolEndpoint |

### SMX_FCHBAConcreteIdentity

SMX_FCHBAConcreteIdentity is used to represent an association between SMX_ComputerSystem and SMX_FCHBAComputerSystem.

| Property Name | Value |
| --- | --- |
| CIM_LogicalIdentity |  |
| CIM_ConcreteIdentity |  |
| SMX_FCHBAConcreteIdentity |  |
| SystemElement | ref:SMX_ComputerSystem |
| SameElement | ref:SMX_ FCHBAComputerSystem |

## Physical Location

The Physical Location is a string representing the physical location of the Fibre Channel Host Bus Adapter device. This string should represent the physical location of the device with which an end-user can uniquely locate the device. Most of these strings will be represented in customer documentation, silkscreen labels, or hood tags.

The following table lists the properties implemented. Any combination of the following applicable descriptors could be used to better define the device location.

| All Systems | HP Integrity Cellular Servers | HP BladeServers in C3000 / C7000 Enclosures |
| --- | --- | --- |
| Slot=<num> | Cabinet=<num> | Blade=<num> |
| Embedded Controller=<logical_ controller_id> | Bay=<num> | RootPort=<num> (if embedded on System Board) |
| System Mainboard (referring to System Board/motherboard) | Chassis=<num> | Mezzanine=<num> (I/O Mezzanine slot num) |

## Provider Indications

| Indications Generated by the Provider | This provider generates no indications. |
| --- | --- |