# HP Insight Manager WBEM Base Server Provider Overview

| Description | The HP Insight Management Web-Based Enterprise Management (WBEM) Base Server provider implements and extends CIM classes described in this document to model the base server. These classes provide computational information and capabilities and serve as an aggregation point to associate one or more elements that make up a base server, such as: Fans, PowerSupplies, Processor and Memory (Volatile and/or NonVolatile Storage). |  |
| --- | --- | --- |
| Profile Name | Organization | Version |
| HP Base Server Profile (P00105) | HP WBEM TC | 1.1.0 |
| HP Base Server Physical Asset Profile (P00114) | HP WBEM TC | 1.1.0 |
| HP Software Inventory (P00127) | HP WBEM TC | 1.0.1 |

| Requirements | For each hardware architecture listed, this provider requires the following distributions HP Integrity managed servers; SLES 10 and later RLEL 5.0 and later. HP ProLiant managed servers; SLES 11 and later RHEL 5.3 and later. |
| --- | --- |
| Release History | Initial release with HP Insight Management WBEM Providers for Linux v2.0. |

## Setting Up the Provider

| Installing the Provider | There are no special installation instructions for this provider. It is installed by default as part of the HP Insight Management WBEM providers. |
| --- | --- |
| Configuring the Provider | This provider does not accept specific configuration adjustments beyond standard HP Insight Management WBEM support. |

## Using the Provider

Namespaces Supported by the Provider This provider returns instances in the root/hpq namespace.

| Schema Supported by the Provider |  |
| --- | --- |
|  | This provider supports the following classes: - SMX_ComputerSystem - SMX_ComputerSystemChassis - SMX_ComputerSystemPackage - SMX_OperatingSystem - SMX_RunningOS - SMX_InstalledOS - SMX_BaseServerECTP - SMX_SMXSoftwareIdentity - SMX_SMXInstalledSoftwareIdentity - SMX_SystemFirmware - SMX_SystemInstalledFirmwareIdentity - SMX_SystemElementFirmwareIdentity |

The tables in the following sections describe the properties of the supported classes. The classes are categorized by the class or superclass that defines the property, the first column is the Property Name (including type and units) and the second column describes how the provider determines the properties implementation. When the Property Implementation value is a number, the number given is the default behavior and the Managed Object Format interpretation is within parenthesis. If other values are returned, a problem is indicated.

Unless otherwise noted, all of the property implementation values given are for HP ProLiant and HP Integrity (cellular and non-cellular) systems. The location related properties and implementation values are determined based on the server type so they may differ.

### SMX_ComputerSystem Class

The SMX_ComputerSystem class implements the HP_ComputerSystem class to model the base server. There is only one instance of SMX_ComputerSystem instantiated on a server.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Computer system DNS name returned by uname. |
| Description | Computer system DNS name returned by uname. |
| ElementName | Computer system DNS name returned by uname. |
| CIM_ManagedSystemElement |  |
| Status | Same as OperationalStatus[0]. |
| OperationalStatus | OperationalStatus[0] contains the computer system consolidated status and indicates the worst status of any connected subsystem. OperationalStatus[0] will contain one of the following: 0 (Unknown) 1 (Other) 2 (OK) 3 (Degraded) 6 (Error) 10 (Stopped) The status values of the subsystems that make up the overall operational status are contained in OperationalStatus[1-n]. See the descriptions for GroupOperationalStatus for the list of possible values. |
| OperationalStatusClassNames | OperationalStatusClassNames[0] contains the computer system class name, SMX_ComputerSystem.CreationClassName. The class name of the HP_GroupSystemSpecificCollection class for the subsystems that make up the overall operational status reported in OperationalStatus[1-n] are contained in OperationalStatusClassNames[1-n]. See the datasheets for each provider for the values of each class name. |
| StatusDescriptions | StatusDescriptions[0] will contain one of the following corresponding to OperationalStatus[0]: One or more of the subsystems that make up this computer system have an OperationalStatus value of Unknown One or more of the subsystems that make up this computer system have an OperationalStatus value of Other One or more of the subsystems that make up this computer system have an OperationalStatus value of Degraded One or more of the subsystems that make up this computer system have an OperationalStatus value of Error One or more of the subsystems that make up this computer system have an OperationalStatus value of Stopped The status descriptions of the subsystems that make up the overall operational status are contained in OperationalStatus[1-n]. These will be set to: Unknown OK Error Stopped Predictive Failure |
| HealthState | 0 (Unknown) 5 (OK) 10 (Degraded) 20 (Major Failure) |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| EnabledDefault | 2 (Enabled) |
| CIM_System |  |
| CreationClassName | SMX_ComputerSystem |
| Roles | Computer system roles string: initialized to not set. |
| CIM_ComputerSystem |  |
| OtherIdentifyingInfo | OtherIdentifyingInfo[0]: <GUID_String> Where: <GUID_String> is the computer system physical GUID string. OtherIdentifyingInfo[1]: <MAC_String> Where: <MAC_String> is the computer system MAC address. OtherIdentifyingInfo[2]: <Model_String>: Where: <Model_String> is the computer system model string; and Serial_Number_String is the computer system physical serial number string. OtherIdentifyingInfo[3]: <Tag_String> Where: <Tag_String> is the computer system asset tag string. OtherIdentifyingInfo[4]: <ID_String> Where: <ID_String> is the computer system physical GUID string. OtherIdentifyingInfo[5]: <ProductID_String> Where: <ProductID_String> is SMX_ComputerSystemChassis.ProductID. OtherIdentifyingInfo[6]: <Target_String> Where: <Target_String> is an opaque string used by the providers. |
| IdentifyingDescriptions | IdentifyingDescriptions[0]: CIM:GUID IdentifyingDescriptions[1]: CIM:MAC IdentifyingDescriptions[2]: CIM:Model:SerialNumber IdentifyingDescriptions[3]: CIM:Tag IdentifyingDescriptions[4] CIM:CorrelatableID IdentifyingDescriptions[5] HPQ:ProductID IdentifyingDescriptions[6] HPQ:SMXTarget |
| Dedicated | 0 (Not Dedicated) |
| HP_ComputerSystem |  |
| Name | Computer system DNS name returned by uname. |
| NameFormat | DNSName |
| PrimaryOwnerName | Computer system primary owner name string; initialized to not set. |
| PrimaryOwnerContact | Computer system primary owner contact string; initialized to not set. |
| PrimaryOwnerPager | Computer system primary owner pager string; initialized to not set. |
| SecondaryOwnerContact | Computer system secondary owner contact string; initialized to not set. |
| SecondaryOwnerName | Computer system secondary owner name string; initialized to not set. |
| SecondaryOwnerPager | Computer system secondary owner pager string; initialized to not set. |
| LocationIndicator | State of the computer system Unit Identifier (UID) LED: 0 (Unknown) 2 (On) 3 (Off) 5 (Alternating) |
| SerialNumber | The computer system physical serial number string. |
| IdentifyingNumber | The computer system physical GUID string. |
| Model | The computer system model string. |
| SMX_ComputerSystem |  |

#### Implemented Methods for SMX_ComputerSystem Class

| Method Name | Method Implementation |
| --- | --- |
| SendTestIndication | Method Parameters: EventID, number of test event to send: 101 - Send Information Test Indication 102 - Send Warning Test Indication 103 - Send Critical Test Indication Return Values: 0 - request to send indication successful 1 - method unsupported 2 - request to send indication failed 3 - provider is busy processing previously requested indication, try again 4 - EventID is not one of the defined values Exception: CIM_ERR_INVALID_PARAMETER - EventID is the wrong type, does not exist or other internal error. |

### SMX_ComputerSystemChassis Class

The SMX_ComputerSystemChassis class implements the HP_ComputerSystemChassis class to model the physical system chassis. There is only one instance of SMX_ComputerSystemChassis instantiated on a server.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_PhysicalElement |  |
| Tag | Unique value for instance. For example: 1 |
| CreationClassName | SMX_ComputerSystemChassis |
| ElementName | Computer system product name (model) string. |
| Manufacturer | Computer manufacturer string. |
| Model | Computer system product name (model) string. |
| SerialNumber | Computer system physical serial number. |
| UserTracking | Computer system asset tag string (writable property; persistent across reboots). |
| Name | Computer system product name (model) string. |
| Status | OK |
| StatusDescriptions | Description of the state of the chassis, for example System is functional. |
| Description | General description of the chassis For example: Physical Package for this system. |
| OperationalStatus | 2 (OK) |
| PoweredOn | TRUE |
| CIM_PhysicalPackage |  |
| RemovalConditions | 2 (Not Applicable) |
| PackageType | 3 (Chassis/Frame) |
| CIM_PhysicalFrame |  |
| CIM_Chassis |  |
| ChassisPackageType | See MOF for definitions. This value is retrieved from SMBIOS. |
| ChassisTypeDescription | See MOF for definitions. This value is retrieved from SMBIOS |
| RackMountable | 0 (Unknown) 1 (True) 2 (False) |
| HP_ComputerSystemChassis |  |
| ProductID | Computer system product ID string For example: 123456-123. |
| VirtualUUID[] | If Synergy is enabled, this will contain the logical UUID for the system. |
| VirtualSerialNumber[] | If Synergy is enabled, this will contain the logical Serial Number for the system. |
| TPMModule | Indicates the installed and enabled state of the Trusted Platform Module. 1: Other; 2: Absent;; 3: Presnet and Enabled; 4: Presnet but Disabled; |
| SMX_ComputerSystemChassis |  |

### SMX_ComputerSystemPackage Class

The SMX_ComputerSystemPackage class implements the HP_ComputerSystemPackage class and associates instances of SMX_ComputerSystem and SMX_ComputerSystemChassis. There is only one instance of SMX_ComputerSystemPackage instantiated on a server.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Dependency |  |
| CIM_SystemPackaging |  |
| CIM_ComputerSystemPackage |  |
| PlatformGUID | Computer system physical UUID value from SMBIOS. |
| HP_ComputerSystemPackage |  |
| Antecedent | References SMX_ComputerSystemChassis |
| Dependent | References SMX_ComputerSystem |
| SMX_ComputerSystemPackage |  |

### SMX_OperatingSystem Class

The SMX_OperatingSystem class implements the CIM_OperatingSystem class to model the system operating system. There is only one instance of SMX_OperatingSystem instantiated on a server.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| ??? | On Linux, first line of /etc/issue. On ESXi, output of vmware -v |
| Description | Output of uname -v. |
| ElementName | Output of uname -s. |
| CIM_ManagedSystemElement |  |
| OperationalStatus | 2 (OK) |
| StatusDescriptions | StatusDescriptions[0] will contain the value map string corresponding to OperationalStatus[0]: For example: OK |
| Status | OK |
| HealthState | 5 (OK) |
| PrimaryStatus | 1 (OK) |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledState | 2 (Enabled) |
| RequestedState | 0 (Unknown) |
| EnabledDefault | 2(Enabled) |
| TransitioningToState | 12 (Not Applicable) |
| CIM_OperatingSystem |  |
| CSName | SMX_ComputerSystem.Name |
| CSCreationClassName | SMX_ComputerSystem.CreationClassName |
| Name | Output of uname -s. |
| CreationClassName | SMX_OperatingSystem |
| Version | Output of uname -r. |
| OtherTypeDescription | On Linux, NULL On ESXi, Embedded |
| OSType | On Linux, 36 (Linux) On ESXi, 39 (VM) |
| LocalDateTime | Current time. |
| LastBootUpDateTime | Time when the operating system was last booted |
| SMX_OperatingSystem |  |

### SMX_RunningOS Class

The SMX_RunningOS class implements the CIM_RunningOS class and associates the SMX_ComputerSystem instance and an SMX_OperatingSystem instance.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_RunningOS |  |
| Antecedent | References CIM_OperatingSystem |
| Dependent | References CIM_ComputerSystem |
| SMX_RunningOS |  |

### SMX_InstalledOS Class

The SMX_InstalledOS class implements the CIM_InstalledOS class and associates the SMX_ComputerSystem instance and a SMX_OperatingSystem instance.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_InstalledOS |  |
| PartComponent | References CIM_OperatingSystem |
| GroupComponent | References CIM_ComputerSystem |
| SMX_InstalledOS |  |

### SMX_BaseServerECTP Class

The SMX_BaseServerECTP class extends the CIM_ElementConformsToProfile class and associates the Base Server registered profile with the HP specific central class that defined the implementation of the profile.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ElementConformsToProfile |  |
| ConformantStandard | References CIM_RegisteredProfile |
| ManagedElement | References CIM_ManagedElement (HP_ComputerSystem for server and network profiles, the storage computer system for storage profiles) |
| HP_ElementConformsToProfile |  |
| SMX_BaseServerECTP |  |

### SMX_SMXSoftwareIdentity Class

The SMX_SMXSoftwareIdentity implements the HP_SoftwareIdentity class to model the SMX software identification information.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Short description of the SMX product For example: HP Standards Management eXtensions (SMX). |
| Description | Description of the SMX product For example: HP Standards Management eXtensions (SMX) are WBEM Providers, using DMTF standard profiles. |
| ElementName | hp-smx |
| CIM_ManagedSystemElement |  |
| OperationalStatus | 2 (OK) |
| StatusDescriptions | Description of the installation status of the software identity For example: SMX is installed properly |
| Status | OK |
| HealthState | 5 (OK) |
| CIM_LogicalElement |  |
| CIM_SoftwareIdentity |  |
| InstanceID | Opaque string using HPQ as OrgID For example: HPQ:SMX_SMXSoftwareIdentity-1. |
| MajorVersion | The first numeric value in the VersionString. |
| MinorVersion | The second numeric value in the VersionString. |
| RevisionNumber | The third numeric value in the VersionString. |
| VersionString | Version of SMX, for example: 02.00.00. |
| Manufacturer | Hewlett Packard Company |
| Classifications[0] | 5 (Instrumentation) |
| IsEntity | TRUE |
| TargetOSTypes[0] | On Linux, 36. On ESXi, 39 (VM). |
| IdentityInfoType[] | IdentityInfoType[0] will contain the following: CIM:SoftwareFamily IdentityInfoType[1] will contain the following: HPQ:SoftwareCategory |
| IdentityInfoValue[] | IdentityInfoValue[0] will contain the following: HPQ:hp-smx IdentityInfoValue[1] will contain the following: System Management |
| HP_SoftwareIdentity |  |
| SMX_SMXSoftwareIdentity |  |

### SMX_SMXInstalledSoftwareIdentity Class

The SMX_SMXInstalledSoftwareIdentity class implements the CIM_InstalledSoftwareIdentity class and associates the SMX_ComputerSystem instance and a SMX_SMXSoftwareIdentity instance.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_InstalledSoftwareIdentity |  |
| InstalledSoftware | References CIM_SoftwareIdentity |
| System | References CIM_ComputerSystem |
| SMX_SMXInstalledSoftwareIdentity |  |

### SMX_SystemFirmware Class

The SMX_SystemFirmware class implements the HP_SoftwareIdentity class and describes the firmware found on the server system.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | A description of the firmware. For example, on HP ProLiant: System ROM Firmware-<family_code>(<state>) () where family_code is the system ROM family code and state is either Active or Redundant. For example: System ROM Firmware-P31 (Active) System ROM Firmware-P31 (Redundant) |
| Description | A description of the firmware. |
| ElementName | A description of the firmware. |
| CIM_ManagedSystemElement |  |
| Name | A name for the firmware. |
| OperationalStatus | 2 (OK) |
| StatusDescriptions | StatusDescriptions[0] will contain the following corresponding to OperationalStatus[0]: System Firmware Status: OK |
| HealthState | 5 (OK) |
| CIM_LogicalElement |  |
| CIM_SoftwareIdentity |  |
| InstanceID | Opaque string using HPQ as OrgID For example: HPQ:SMX_SystemFirmware:<n> Where: <n> is used to identify the instance. For example: HPQ:SMX_SystemFirmware:1 |
| MajorVersion | The firmware major version. |
| MinorVersion | The firmware minor version. |
| VersionString | The firmware version. |
| Manufacturer | HP |
| Classifications | 11 (Bios/FCode) |
| ClassificationDescriptions | System Firmware |
| IdentityInfoValue | On HP ProLiant, this will contain: HPQ:<family_code> Where: <family_code> is the system ROM family code. For example: HPQ:P31 On HP Integrity, this will contain: HPQ:ElementName |
| IdentityInfoType | CIM:SoftwareFamily |
| ReleaseDate | YYYYMMDD******.********** Where: YYYY is the 4 digit year, MM is the 2 digit month, and DD is the 2 digit day |
| SMX_SystemFirmware |  |

### SMX_SystemInstalledFirmwareIdentity Class

The HP_SystemInstalledFirmwareIdentity class extends the CIM_InstalledSoftwareIdentity class and associates the HP_ComputerSystem instance and a SMX_SystemFirmware instance.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_InstalledSoftwareIdentity |  |
| SMX_SystemInstalledFirmwareIdentity |  |
| InstalledSoftware | References SMX_SystemFirmware |
| System | References HP_ComputerSystem |

### SMX_SystemElementFirmwareIdentity Class

The HP_SystemElementFirmwareIdentity class implements the HP_ElementSoftwareIdentity class and associates the HP_ComputerSystem instance and SMX_SystemFirmware instances.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Dependency |  |
| CIM_ElementSoftwareIdentity |  |
| UpgradeCondition | 3 (Owner Upgradeable) |
| ElementSoftwareStatus | The ElementSoftwareStatus array will contain the following if the Antecedent refers to an instance of an Active Firmware: ElementSoftwareStatus[0]: 6 (Installed) ElementSoftwareStatus[1]: 2 (Current) The ElementSoftwareStatus array will contain the following if the Antecedent refers to an instance of a Redundant Firmware: ElementSoftwareStatus[0]: 6 (Installed) ElementSoftwareStatus[1]: 4 (Fallback) |
| SMX_SystemElementFirmwareIdentity |  |
| Antecedent | References SMX_SystemFirmware |
| Dependent | References HP_ComputerSystem |

## Supported Indications

The following table describes the properties that are common to all of the SMX Base Server Provider indications that are implemented for server platforms, where available.

The following table lists the common properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of the indication |
| IndicationTime | Time of indication |
| CIM_AlertIndication |  |
| EventTime | Time of the event or time of the indication if event time unknown |
| SystemName | SMX_ComputerSystem.Name |
| SystemCreationClassName | SMX_ComputerSystem.CreationClassName |
| HP_AlertIndication |  |
| ProviderVersion | Provider Version in the format VV.UU.FF. For example: 01.05.00 |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication |
| OSType | On ESX, 39 (VM) On Linux, 36 (Linux) |
| OSVersion | The operating system version of the computer system generating the indication in the following format: <major>.<minor>.<build> |
| SystemFirmwareVersion | Array of firmware versions of the computer system generating the indication |
| SystemSerialNumber | Serial number of the computer system generating the indication |
| SystemProductID | Product ID of the computer system generating the indication |
| SystemModel | Model name of the computer system generating the indication |
| SystemGUID | Platform GUID of the computer system generating the indication |

### HP_DeviceIndication:WBEM Providers Started

The following tables describe the SMX Base Server Provider indications that are implemented for all server platforms.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 2 (Informational) |
| CIM_AlertIndication |  |
| Description | SMX WBEM Providers have started |
| AlertType | 5 (Device Alert) |
| EventID | 1 |
| ProviderName | HP WBEM |
| RecommendedActions | No action is recommended. |
| HP_AlertIndication |  |
| Summary | WBEM Providers started |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | WBEM Providers Started |

### HP_DeviceIndication: WBEM Providers Informational Test Indication

| Property Name | Property Implementation |  |
| --- | --- | --- |
| CIM_Indication |  |  |
| PerceivedSeverity | 2 (Informational) |  |
| CIM_AlertIndication |  |  |
| Description |  | This is an information test event. |
| AlertType |  | 5 (Device Alert) |
| EventID |  | 1 |
| ProviderName |  | HP Test |
| RecommendedActions |  | No action is recommended. |
| HP_AlertIndication |  |  |
| Summary | Test Event |  |
| ProbableCause | 1 (Other) |  |
| ProbableCauseDescription | Test Indication |  |

### HP_DeviceIndication: WBEM Providers Warning Test Indication

| Property Name | Property Implementation |  |
| --- | --- | --- |
| CIM_Indication |  |  |
| PerceivedSeverity | 3 (Warning) |  |
| CIM_AlertIndication |  |  |
| Description |  | This is a warning test event. |
| AlertType |  | 5 (Device Alert) |
| EventID |  | 2 |
| ProviderName |  | HP Test |
| RecommendedActions |  | No action is recommended. |
| HP_AlertIndication |  |  |
| Summary | Test Event |  |
| ProbableCause | 1 (Other) |  |
| ProbableCauseDescription | Test Indication |  |

### HP_DeviceIndication: WBEM Providers Critical Test Indication

| Property Name | Property Implementation |  |
| --- | --- | --- |
| CIM_Indication |  |  |
| PerceivedSeverity | 6 (Critical) |  |
| CIM_AlertIndication |  |  |
| Description |  | This is a Critical Test Event |
| AlertType |  | 5 (Device Alert) |
| EventID |  | 3 |
| ProviderName |  | HP Test |
| RecommendedActions |  | No action is recommended. |
| HP_AlertIndication |  |  |
| Summary | Test Event |  |
| ProbableCause | 1 (Other) |  |
| ProbableCauseDescription | Test Indication |  |

### HP_DeviceIndication: ASR Reboot Occurred

| Property Name | Property Implementation |  |
| --- | --- | --- |
| CIM_Indication |  |  |
| PerceivedSeverity | 5 (Major) |  |
| CIM_AlertIndication |  |  |
| Description |  | The system has rebooted from an Automatic Server Recovery (ASR) event. |
| AlertType |  | 5 (Device Alert) |
| EventID |  | 1 |
| ProviderName |  | HP Recovery |
| RecommendedActions |  | Determine the nature of the ASR event and take corrective action. |
| HP_AlertIndication |  |  |
| Summary | ASR reboot occurred |  |
| EventCategory | 16 (System Power) |  |
| ProbableCause | 111 (Timeout) |  |
| ProbableCauseDescription | ASR reboot occurred |  |

### HP_DeviceIndication: Thermal Reboot Occurred

| Property Name | Property Implementation |  |
| --- | --- | --- |
| CIM_Indication |  |  |
| PerceivedSeverity | 5 (Major) |  |
| CIM_AlertIndication |  |  |
| Description |  | The system has rebooted from thernal shutdownevent. |
| AlertType |  | 5 (Device Alert) |
| EventID |  | 2 |
| ProviderName |  | HP Recovery |
| RecommendedActions |  | Replace any failed or removed fans. Insure the air flow into, out of, and within the system chassis is not obstructed. |
| HP_AlertIndication |  |  |
| Summary | Thermal reboot occurred |  |
| EventCategory | 23 (System Cooling) |  |
| ProbableCause | 51 (Temperature Unacceptable) |  |
| ProbableCauseDescription | Thermal reboot occurred |  |

### HP_DeviceIndication: Fan Failure Reboot Occurred

| Property Name | Property Implementation |  |
| --- | --- | --- |
| CIM_Indication |  |  |
| PerceivedSeverity | 5 (Major) |  |
| CIM_AlertIndication |  |  |
| Description |  | The system has rebooted from fan failure shutdown event. |
| AlertType |  | 5 (Device Alert) |
| EventID |  | 3 |
| ProviderName |  | HP Recovery |
| RecommendedActions |  | Replace any failed or removed fans. |
| HP_AlertIndication |  |  |
| Summary | Fan failure reboot occurred |  |
| EventCategory | 23 (System Cooling) |  |
| ProbableCause | 94 (Fan Failure) |  |
| ProbableCauseDescription | Fan failure reboot occurred |  |

### HP_DeviceIndication: VRM Failure Reboot Occurred

| Property Name | Property Implementation |  |
| --- | --- | --- |
| CIM_Indication |  |  |
| PerceivedSeverity | 5 (Major) |  |
| CIM_AlertIndication |  |  |
| Description |  | The system has rebooted from VRM failure shutdown event. |
| AlertType |  | 5 (Device Alert) |
| EventID |  | 4 |
| ProviderName |  | HP Recovery |
| RecommendedActions |  | Replace any failed or removed power supplies. |
| HP_AlertIndication |  |  |
| Summary | VRM failure reboot occurred |  |
| EventCategory | 16 (System Power) |  |
| ProbableCause | 89 (Power Supply Failure) |  |
| ProbableCauseDescription | VRM failure reboot occurred |  |

### HP_DeviceIndication: POST Occurred

| Property Name | Property Implementation |  |
| --- | --- | --- |
| CIM_Indication |  |  |
| PerceivedSeverity | 5 (Major) |  |
| CIM_AlertIndication |  |  |
| Description |  | Power-On Self-Test errors occurred during the last system startup. |
| AlertType |  | 5 (Device Alert) |
| EventID |  | 1 |
| ProviderName |  | HP Post |
| RecommendedActions |  | Determine the nature of the POST error and take corrective action. |
| HP_AlertIndication |  |  |
| Summary | POST errors occurred |  |
| EventCategory | 4 (System Harware) |  |
| ProbableCause | 8 (Configuration/Customization Error) |  |
| ProbableCauseDescription | POST Errors occurred |  |
| VariableName | VariableNames[0]: POST Error Code VariableNames[1]: POST Error String |  |
| VariableType | VariableTypes[0]: 3 (uint8) VariableTypes[1]: 1 (string) |  |
| VariableValue | VariableValues[0]: The POST error code as specified by the System ROM VariableValues[1]: The POST error string as specified by the System ROM |  |